# Transfer Function Analysis and VibMScope Framework

# 1. Overview

This document presents a complete framework for simulating, capturing, and analyzing vibration transmissibility using a modular Python-based system named VibMScope™. The framework includes real-time support, sweep simulation, FFT-based transmissibility extraction, and professional report generation including metadata, plots, and summary tables.

# 2. Objective

To develop a consistent and extensible system for analyzing transmissibility (output/input vs frequency) in vibration systems, with support for:

- Simulation mode (chirp/sweep)

- Live sensor capture (remote streaming)

- FFT-based analysis (amplitude and phase)

- Professional output (.pdf, .csv, .txt, .npz)

- Accurate metadata recording

# 3. Architecture Summary

The architecture includes components for simulation, post-processing, GUI integration, and metadata export. Data is stored in .npz format with full traceability.

# 4. Methodology

The system uses FFT-based amplitude ratio for transmissibility extraction, rejecting the less robust time-domain ratio method. Frequency-domain comparison ensures better accuracy.

# 5. Transmissibility vs Transfer Function

Case A (Force → Acceleration):

TF_A(f) = sqrt{ [1 + (2ζf/fn)^2] / [(1 - (f/fn)^2)^2 + (2ζf/fn)^2] }

Case B (Displacement input/output):

T(f) = 1 / sqrt{ (1 - (f/fn)^2)^2 + (2ζf/fn)^2 }

# 6. System Parameter Estimation

Parameters like damping ratio, natural frequency, and system order are extracted via model fitting and peak-counting. Line styles are used instead of colors to ensure print-safe legend.

# 7. Output Formats

.npz for raw signals + metadata

.txt for aligned field summary

.pdf for TF plots and embedded metadata

.csv optionally for TF curve

# 8. Metadata and Reproducibility

Each output contains data capture info, system info, processing method, and tool version — supporting traceability and repeatability.

# 9. GUI Integration (VibMScope)

The GUI is implemented using Tkinter and Matplotlib, includes responsive dual-pane display, DPI awareness, and shortcut keys.

# 10. Design Strengths

- Modular, Extensible

- FFT-only method for robustness

- Monochrome-safe output

- Unified handling of live and simulated modes

# 11. Publication Readiness

This framework is suitable for IEEE conference papers, technical white papers, or internal engineering documentation. Ready for LaTeX and Word export.